BOROUGH GREEN SANDPITS & LANDFILL SITES REPORT

Tonbridge & Malling Borough Council's Local Plan depends largely on building 3000 houses on Metropolitan Greenbelt and the Kent Downs Area of Outstanding Natural Beauty land to the north of Borough Green Village,

TMBC's justification for development on MGB/AONB land is their Local Plan Building Block Provision of Infrastructure in the form of a "Relief Road" stretching from Darkhill Roundabout on the A25 to the West, and the A20 at Nepicar in the East.

They have made some commitment to ensuring the Relief Road is in place during to housing development, and we will be insisting on an ironclad agreement between multiple landowners to fund that road.

However, we believe there are significant obstacles to be overcome, many of which mean that the 1720 houses and the Relief Road cannot be achieved within the timeframe of this Local Plan prior to 2031, and therefore render the plan unsound.

1. Ightham Sandpit requires a huge amount of contamination remediation, estimated at 200,000 tonnes plus.

2. Borough Green Sandpit has significant reserves still to be exploited, most notably in the land north of the existing workings and the M26.

3.Park Farm Claypit has extant sand and clay extraction permission until 2042.

4 Nepicar Sandpit has extant extraction permissions until 2028, plus infill. It also has legal problems over access to residential properties that could add a further 10-20 years to the scheme.

5. H+H Celcon has HGV number restrictions - remediation access to Ightham Sandpit will severely impact the viability of the Block Factory.

6. Borough Green Sandpit and Nepicar Sandpit have HGV number restrictions that will inhibit faster extraction and subsequent landfill.

7. Ightham Sandpit, Borough Green sandpit and Park Farm Claypit, and the Nepicar pit each separately block the site north to south, the route of the Relief Road through the site. Each needs to substantially completed before the Relief road and the MGB justification can be realised.

8. HGV movements to finish excavation and landfill the sites will have a severe impact on traffic levels and the AQMA that the scheme purports to be resolving.

9. Landfill space nationally is severely constrained, and recycling in landfill sites is a crucially important element.

10. Building Sand is a nationally Strategic Mineral, and particularly important to housebuilding at this time. Current reserves run out in 2023.

11. There is no evidence to suggest that even if HGV numbers were unrestricted, that the market could receive the volumes of sand involved, nor supply the necessary infill material, even if recycling were abandoned.

It must be borne in mind that no local Quarry or Sandpit has ever completed extraction and restoration within the first permitted time frame, they always apply for re-extensions, and many of these estimates are based of pre-recycling policies.

IGHTHAM SANDPIT    Rear of H+H Celcon Factory Contaminated landfill

BOROUGH GREEN LANDFILL  ( Wrotham Rd, Part Historic Landfill still operating today)

BOROUGH GREEN SANDPIT  (Off Platt Ind Est, Maidstone Rd, Part Historic Landfill still operating)

PARK FARM QUARRY (CLAYPIT) North of Borough Green Sandpit

NEPICAR SANDPIT  East of Platt Ind Est, on Maidstone Rd Platt

JOCO PIT (Wrotham Rd, now Tolsey Mead and Eaglestone Close)

ISLES QUARRY WEST (off Darkhill Road, now Hazelbourne Ave, Isles Quarry Rd, Bangays Way)

A-Z Site (Off Fairfield Rd, Formerly Novellos and A-Z factories, now Talbot Way and Pearsall Way)

These last three sites are not within the Borough Green Garden City proposal, but we need to learn from the mistakes made during and since development of these completed sites, to ensure the lessons learned are carried forward and the BGGC proposal protects the new residents, water supplies and the surrounding area.

We need more robust Local Plan Policies on Construction on Historic Landfill Sites, Contamination Remediation, and Developer Viability, and these are discussed at greater length in the attached Landfill Report.

CURRENT LANDFILL MAP JUNE 2018

IGHTHAM SANDPIT

North East of the H+H Celcon Plant, and owned by them.

This land was excavated for sand by Kent White Brick from the 1950s, probably finishing about 1990. The land was then  operated by H+H Celcon Ltd under TM/87/346.

Landfill was carried out from 1991 to 2000, ostensibly using hassock from Stangate and Isles Quarries, but very little is known about where the fill really came from, and the EA and KCC seem unaware of any landfill permission or monitoring. However, there are strong suspicions, supported by anecdotal evidence, that the fill came from the BP Isle of Grain terminal, and the Greenwich Gas Works , when these sites were decommissioned and decontaminated.

This supposition is strongly supported by the 2006 ES Addendum, a study carried out for the Celcon2 Factory application in 2005 TM/03/02563, which collapsed at Inquiry in 2007. This site investigation highlights large amounts of oily contaminated material, plastic, brick and concrete, pipework, steel, and drums of unknown contents.

H+H Celcon have a current variation application to import Sand from elsewhere for the block making operation.

HGVs restricted to 340 per day, 170m each way

Attached Evidence:

2006 Southern Testing Ltd "Site Investigations and Risk Assessment Addendum" (TM/03/02563 inquiry)

2018 Local Plan Submission Wrotham Parish Council et al

2006 Barton Willmore covering letter (inquiry evidence.

Decision TM/87/346

1984 Ightham Sandpit already shown as disused

(Extract    1929)

Aerial View Ightham Sandpit June 2018 looking East

Borough Green Landfill immediately beyond that,

and Borough Green and Nepicar Sandpits in the distance

BOROUGH GREEN LANDFILL  (Wrotham Road, Borough Green)

(FORMERLY TALBOT SANDPIT, also known as Borough Green Sandpit Wrotham Rd),

Original permission TM/69/10748 to excavate sand to the north west of Borough Green from Wrotham Road to the Ightham sandpit boundary, bounded to the north by the route of the proposed Borough Green Bypass.

TM/06/01271 allowed a recycling operation to be added to the Hall Aggregates landfill operation subsequently operated by Cemex, until taken over by Robert Body Haulage. Some sand is still being excavated in the north west corner.

It is recorded that a sandpit already existed at the station site in the early 1800s, (See A-Z), but  excavation began in earnest here as a result of the building of the Railway Line that opened in 1874, using the material to build the embankments East and West of the village.

Sand excavation moved north and east from this point, following the line of the Wrotham Road (A227) behind Longpond cottages, as far as the Pottery Works where Potters Mede is now. Once they reached that northerly point, excavation moved west following the rear of what  became Fairfield Estate, until they met the Kent White excavations at Ightham Sandpit coming east, the boundary between the two pits becoming MR244. The area immediately west of Wrotham Rd, and the area north of Fairfield (built 1948) were landfilled with London and local domestic waste.

Whilst Robert Body carries out the current landfill operation under strict modern rules, the two areas mentioned (see map) will require special investigation and landfill gas monitoring.

Significant landfill still required to attain local levels. Under TM/14/02728 the approval is in force until 2042, but with HGVs restricted to 182 per day, 92 in and 91 out.

Evidence

TM/69/10748   Decision (below)

TM/06/01271   Decision

TM/14/ 02728  Decision

Extract 1897 Station Yard                                                                              1961 Extended to Wrotham Road A227

BOROUGH GREEN LANDFILL  JUNE 2018

BOROUGH GREEN SANDPIT

(FORMERLY SMITHS SANDPIT, also known as Borough Green Sandpit, Platt)

This was originally opened up as a series of claypits supplying local potters and brickworks, already in existence on the earliest maps we have from 1870. Platt Industrial  Estate is built on the site of the earliest claypit and brickworks, with further excavations still evident as ponds and pits today, to the north of the Industrial Estate. Botany Woods still contains foundations of another brickworks. At some point sand was discovered under the clay.

Sand excavations began pre-war and moved west from the Industrial Estate, and south towards the Railway. This site is a little more difficult to establish a history, but it was also landfilled in the past before regulation, and is also likely to suffer from non-inert waste and landfill gas.

The current operations are carried out under a series of permissions, TM/71/11061. TM/07/00512 and TM/09/02028. The sand extraction is now augmented by soil reclamation as part of the landfill operation, and the operation extended until 2025. There is an HGV restriction to 60 hgv/day, 30 each way.

Evidence:

TM/71/11061

TM/07/00512

TM/09/02028   (there are other interim variation applications)

1870 existing early pits

BOROUGH GREEN SANDPIT North Platt Ind Est June 2018

Borough Green Sandpit Landslip - PROW - 23 Sept 2018

PARK FARM CLAYPIT

(OPERATED BY RUGBY CEMENT TO SUPPLY HOLBOROUGH CEMENT WORKS)

This pit, north of Borough Green and Nepicar Sandpits, has been disused for some years, but unrestored. There is planning permission for a landfill access haul road through Nepicar Sandpit, but the existing access is via Platt Industrial Estate. The site has an approval under TM/ 54/10547 (1947) for the extraction of clay and sand (Pascalls Brickyard).TM/76/10955, TM/91/51708 and TM/02/02663 allowed clay extraction for the Rugby Cement Works at Halling, and extend clay and sand extraction until 2040 restoration by 2042. There is also a permission for a new access road through Nepicar Sandpit

Evidence :  Decisions TM/ 54/10547(1947), TM/76/10955, TM/91/51708 and TM/02/02663

Park Farm Quarry Claypit June    2018

Site Boundary Plan TM/08/02869 -The south western boundary extends across the top of Borough Green Sandpit

NEPICAR SAND QUARRY  ( J CLUBB 1993)

Originally permitted in 1993 as a supply for Ryarsh Brick works, the permission was modified in 1998 to allow general sand sales. Clubbs recently purchased Nepicar Farm to the northeast of the existing pit. Phase 4 extraction 2 years from 2028, Clubbs estimate 2.3m tonnes of sand to extract. There is much useful information in the March 2016 Civitas Planning report in TM/15/01636, which notes problems diverting the access road through Nepicar Farm because Nepicar  Meadow Cottage and  The Chalet who have access rights over the road refuse their permission for the diversion. Accommodating this could take "20 years from today", long past the requested completion of excavation in Phase 4 of 2028/2030, and therefore extending the time frame well beyond the current 2031 envisaged by the Local Plan.

The decisions refer to a daily HGV traffic of 94 per day , falling to 68, 34 each way.

Evidence

TM/98/01815  Decision

TM/ 08/02653  Decision

TM/ 15/01636  Decision

Civitas Report TM/15/01636

Site Plan TM/15/01636

Current picture facing West June 2018 Landfill status unknown

Extract Civitas Planning Report TM/15/01636

3.12 Figure Two below shows the proposed new phasing which includes the

increased extraction area gained by remodelling the quarry. Analysis suggests the

remodelled phasing is better able to provide the sand grades for blending to meet

customer demand. Clearly there is a better proportion of sand colour in the new

phases which is achieved by increasing the size of the phase to provide sufficient

sand of the required grade. The re-modelling also maximises the sand that can be

won, increasing volumes from 1.9mt to 2.3mt with the new phases.

Figure Two Sand Volumes by Proposed Phasing

| Phase | Brown | Pale | Orange | Total(tonnes) |
| --- | --- | --- | --- | --- |
| 1 | 288,600 | 650,600 | 39,500 | 978,700 |
| 2 | 79,400 | 163,400 | 31,700 | 274,500 |
| 3 | 337,600 | 364,000 | 53,900 | 755,500 |
| 4 | - | 134,100 | 170,900 | 305,000 |
| Total | 705,600 | 1,312,000 | 296,000 | 2,313,700 |

3.13 Current production (2013 sales) comprised some 162,000 tonnes of which

96,000t was of blended products and 66,000t unmixed products and subject to

market forces this is anticipated to be maintained. At this rate the new phases would

be extracted as follows:

Figure Three Indicative Phase Duration

| Phase | Commence | Duration (years) |
| --- | --- | --- |
| 1 | 2015 | 6 |
| 2 | 2021 | 2 |
| 3 | 2023 | 5 |
| 4 | 2028 | 2 |

==========================================================================

KCC Minerals & Waste Local Plan

SOURCE        KCCMWLPCS023-12.pdf                   Table 18 P39

Table 18 - Landbank Calculations for Land-won Sand and Gravel for the Plan Period

| YEAR | RESERVES at Start of Year (mt) | DRAWDOWN during Year (mt) | RESERVES at YEAREND | RESERVES Required for 7 year Landbank |
| --- | --- | --- | --- | --- |
| 2012 | 18.674 | 1.603 | 17.071 | 0.00 |
| 2013 | 17.017 | 1.603 | 15.486 | 0.00 |
| 2014 | 15.468 | 1.603 | 13.865 | 0.00 |
| 2015 | 13.865 | 1.603 | 12.262 | 0.00 |
| 2016 | 12.262 | 1.603 | 10.659 | 0.561 |
| 2017 | 10.659 | 1.603 | 9.056 | 2.164 |
| 2018 | 9.056 | 1.603 | 7.453 | 3.767 |
| 2019 | 7.453 | 1.603 | 5.850 | 5.370 |
| 2020 | 5.850 | 1.603 | 4.247 | 6.973 |
| 2021 | 4.247 | 1.603 | 2.644 | 8.576 |
| 2022 | 2.644 | 1.603 | 1.041 | 10.179 |
| 2023 | 1.041 | 1.603 | -0.562 | 11.782 |
| 2024 | -0.562 | 1.603 | -2.165 | 13.385 |
| 2025 | -2.165 | 1.603 | -3.768 | 14.988 |
| 2026 | -3.768 | 1.603 | -5.371 | 16.591 |
| 2027 | -5.371 | 1.603 | -6.974 | 18.194 |
| 2028 | -6.974 | 1.603 | -8.577 | 19.797 |
| 2029 | -8.577 | 1.603 | -10.180 | 21.400 |
| 2030 | -10.180 | 1.603 | -11.783 | 23.003 |

8.1.2 KCC had earlier proposed to plan for an additional 10% of land-won sand and gravel over and above the minimum amount required by the apportionment figure. However, it was explained in the Mineral Sites 'Preferred Options' consultation document (May 2012) that it was not possible to identify sufficient deliverable sustainable sites to make provision for the landbank requirement for the whole of the plan period plus an additional 10%. This remains the case. The Mineral Sites plan will make provision for sufficient aggregate for the plan period based upon calculations using the 10 year rolling average sales figure.

8.1.3 Both the Sustainability Appraisal commentary report and the majority of responses from the Core Strategy 'Issues' consultation (September 2010), supported the provision of an additional 10% over and above the required land bank levels, if conditions allowed. However, at the next consultation stage, the Strategy and Policy Directions consultation for the Core Strategy (May 2011), responses on this issue

were more balanced with five people supporting the additional 10% and four people objecting to it, with two people only commenting.

=====================================================================================

Kent Minerals and Waste Local Plan 2013-30 Adopted July 2016

Kent County Council

5.2.34 Silica sand is used in a range of applications including the manufacture of

glass and production of materials used in construction. An example of a potential local

use would be in the manufacture of ‘Aircrete’ blocks (also known as aerated concrete

blocks) where it may substitute for the current supply of Pulverised Fuel Ash (PFA).

Currently the existing market need for silica sand is being met by extraction from two

quarries Wrotham Quarry (Addington Sand Pit) and Nepicar Sand Pit. These have

permitted reserves in the region of 2.1 mt. These quarries are identified in Appendix C

and shown in Figure 13: Minerals Key Diagram. Wrotham Quarry site has a potential

extension area but that lies within the Kent Downs AONB. While the Plan seeks to

maintain a stock of permitted reserves, in line with national policy, it is recognised that

this may not be possible if it would be inconsistent with policy to conserve the landscape

and scenic beauty of the AONB. In light of national policy, the Plan does not seek

allocation of sites within the AONB or in locations which would have an adverse impact

on the setting of, and implementation of, the statutory purposes of the AONB. Proposals

will be considered on their merits against policy CSM 2.

5.5.6 Geological mapping is indicative of the existence of a mineral resource. It is

possible that the mineral has already been extracted and/or that some areas may not

contain any of mineral resource being safeguarded. Nevertheless, the onus will be on

promoters of non-mineral development to demonstrate satisfactorily(64) at the time that

the development is promoted that the indicated mineral resource does not actually exist

in the location being promoted, or extraction would not be viable or practicable under

the particular circumstances.

5.5.9 Economic land-won minerals that are identified for safeguarding in Kent are

sharp sand and gravel, soft sand, silica sand, crushed rock, building stone and brickearth.

As chalk and clay (other than brickearth) are abundant across the county, they are not

being safeguarded. The mineral resource areas identified for safeguarding are shown

in the MSAs in Chapter 9: Adopted Policies Maps. The MSAs are based on mapping of the mineral resource prepared by the BGS. Current guidance advises that mineral

safeguarding should not be curtailed by any other planning designation, such as

environmental designations without sound justification. The mineral resources within

the Plan area are extensive and whilst they continue beneath urban areas they are

already sterilised by non-mineral development with very little prospect of future working.

Therefore in order for the safeguarding to be practical such areas have been excluded

from the MSAs.

Mineral Reserves in Kent without Confidentially Millions of tonnes (mt) (1)

Restrictions

| Soft Sand | 10.6 mt |
| --- | --- |
| Soft Sand (non-aggregate uses including silica or industrial sand) | 2.2mt |
| Sharp Sand and Gravel | 3.61mt |
| Sharp Sand and Gravel (non-aggregate uses) | 0.50 |
| Chalk (for agricultural uses) | 1.9mt |

1.